PATENTABILITY SEARCH REPORT

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

ReBolt™ Security Adapter

Permanent Plug Attachment System with Telescopic Security Pin
and Controllable Release Mechanism

Date: April 26, 2026

Prepared for: ReBolt™ Inventor / Patent Counsel

Subject: Two Novel Features of the ReBolt™ Security Adapter

# Section 1 — Client Instructions

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

The client has disclosed a security adapter device (ReBolt™) comprising two principal novel features: (1) a permanent adapter-to-plug attachment system utilizing a dual-mechanism approach combining a mechanical ratcheting clamp (zip-tie style) that cinches around the prong base with a medical-grade or industrial-grade adhesive applied to the mating surface between the adapter and the charger body, rendering the attachment irreversible without destruction; and (2) a telescopic security pin housed within the permanently mounted adapter that automatically extends from the adapter face upon removal of the plug from an electrical outlet, physically blocking the prong faces and preventing re-insertion, with the pin retracting only upon successful activation of a user-controlled combination code or push-pin mechanism. A comprehensive prior art search has been conducted to identify any existing patents, patent applications, or non-patent literature disclosing these features individually or in combination, across all classes of plugs and chargers including AC power adapters, USB-C chargers, industrial plugs, and power tool chargers. International prior art has been included.

# Section 2 — Databases Used

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

| Category | Databases Searched |
| --- | --- |
| Patent Databases | Google Patents ¦ USPTO Patent Public Search ¦ Espacenet (worldwide.espacenet.com) ¦ Free Patents Online ¦ The Lens (lens.org) |
| Non-Patent Databases | Google Scholar ¦ IEEE Xplore ¦ arXiv ¦ Master Lock Product Catalog ¦ Brady Safety Product Catalog ¦ Lock Socket / iCubeLock Commercial Products ¦ NFPA 70E Standards ¦ Kickstarter ¦ Indiegogo ¦ Amazon ¦ Grainger |

# Section 3 — Conclusion

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

This patentability search was conducted to identify prior art relevant to the ReBolt™ Security Adapter, which comprises a permanent adapter-to-plug attachment system using a combined ratcheting clamp and adhesive mechanism, and a telescopic security pin with automatic deployment and combination-code-controlled retraction. The search covered United States patents and published applications, international patents (including Chinese, Japanese, Korean, German, and European filings), and non-patent literature including academic publications, industry standards, and commercial products. The multi-pass search methodology included keyword searches, CPC/IPC classification mining, forward and backward citation chaining, and foreign/NPL literature review. The search identified substantial prior art in the fields of plug lockout devices, plug retention adapters, and electrical plug security mechanisms. However, the specific combination of a permanently bonded ratcheting adapter with an automatically deploying telescopic pin requiring a combination code for retraction appears to occupy a distinct position in the prior art landscape. Individual elements of the invention find varying degrees of representation across the identified references.

# Section 4 — Key Features

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

KF 1: A third-party security adapter device configured for permanent, irreversible attachment directly to the male prongs of a standard AC power plug or charger block.

KF 1A: The permanent attachment utilizes a mechanical ratcheting clamp system (zip-tie style) that cinches around the prong base.

KF 1B: The permanent attachment further utilizes a medical-grade or industrial-grade adhesive applied to the mating surface between the adapter body and the charger body.

KF 1C: The combination of mechanical ratcheting clamp and adhesive renders the adapter non-removable without destruction of the adapter or plug.

KF 2: A telescopic pin mechanism housed within the permanently mounted adapter body.

KF 2A: The telescopic pin extends automatically from the face of the adapter upon removal of the plug from an electrical outlet.

KF 2B: The extended pin physically blocks the prong faces, preventing re-insertion of the plug into any electrical outlet.

KF 2C: The pin retracts only upon successful activation of a user-controlled release mechanism.

KF 3: The release mechanism comprises either (a) a push-button or push-pin combination code entered simultaneously, or (b) a rolling combination-lock dial mechanism.

KF 4: The device combines Features 1 and 2 in a single integrated adapter unit.

KF 5: The device is applicable across all classes of plugs and chargers including AC power adapters, USB-C chargers, industrial plugs, and power tool chargers.

# Section 5 — Details of Identified Citations

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

## 5.1 Patent Literature

| S.No. | Publication No. | Title | Assignee(s) |
| --- | --- | --- | --- |
| 1 | US4579410A | Security Attachment for Electrical Plug | Leonard Soloman (Individual) |
| 2 | US5911586A | System, Device and Method for Locking and Unlocking Power Flow to an Electrical Cord | H. Peter Wintergerst (Individual) |
| 3 | US7497705B2 | Lockout Device | Master Lock Co LLC |
| 4 | US10483694B2 | Lock-in Securable Electrical Plug Adapter and Method of Use | Robert W. Crowder (Individual) |
| 5 | US7077683B1 | Ratcheting Electrical Plug and Receptacle | Nicky Charles Ross (Individual) |
| 6 | US5071360A | Permanently Attachable Key-Activated On/Off Switch | Edgar J. Lindow et al. (Individual) |
| 7 | US11824309B2 | Device and Method for Locking Electrical Plugs to Power Outlets | Mohamed Abouismail (Individual) |
| 8 | US6419504B1 | Slide Locked Retractable Grounding Pin Power Cord Plug | Richard Bryant Nelson (Individual) |

## 5.2 Non-Patent Literature

| S.No. | Title | Author(s) / Manufacturer | Publication Date |
| --- | --- | --- | --- |
| 1 | Master Lock S2005 — Compact Plug Prong Lockout | Master Lock Co LLC | Current (Active Product) |
| 2 | Lock Socket — Charger Security Device | Lock Socket Inc. (Shaant Varjabedian) | Current (300K+ units sold) |
| 3 | NFPA 70E — Standard for Electrical Safety in the Workplace | National Fire Protection Association | 2021/2027 Edition |

# Section 6 — Patent and Non-Patent Citations (Detailed Write-Ups)

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

## Reference 1 — US4579410A

This early patent discloses a security attachment that plugs between an electrical plug and a wall receptacle. A first adapter is connected to the plug such that they 'cannot become unplugged,' using a one-way screw or bracket that effectively creates a permanent bond. When the second adapter is removed, the first adapter exposes mismatched prongs that physically block insertion into a standard outlet — directly relevant to both ReBolt's permanent attachment concept (KF 1) and prong-face blocking concept (KF 2B).

| Patent No. | US4579410A (https://patents.google.com/patent/US4579410A) |
| --- | --- |
| Title | Security Attachment for Electrical Plug |
| Filing Date | January 10, 1985 |
| Publication Date | April 1, 1986 |
| Earliest Priority | February 15, 1983 (CIP of US4494809) |
| Inventor(s) | Leonard Soloman |
| Assignee — Original | Leonard Soloman (Individual) |
| Family Members | US4494809A (parent) |
| CPC / IPC | H01R13/6397 ¦ H01R31/06 |

Relevant Excerpts:

• A security attachment for an electrical plug comprising "adapter elements which are mutually plugged together between the electrical plug and a wall receptacle."  ~Abstract

• The first adapter is "connected together [with the plug] such that they cannot become unplugged if an attempt is made to unplug them."  ~Abstract

• A bracket connector encircles the insulated wire, fastened with a "one-way drive head" screw that only threads in, creating a permanent, destructive-removal-only bond.  ~Detailed Description

• The first adapter exposes "prongs which prevent the first adapter from being plugged into the wall receptacle" when the second adapter is removed.  ~Abstract

• The mismatched terminal configuration (one flat blade plus one round pin) "preclude[s] establishing circuit continuity" with a standard receptacle.  ~Claims

• The screw head "can be countersunk so that its existence is not readily apparent, discouraging children from disconnecting."  ~Detailed Description

## Reference 2 — US5911586A

This patent discloses an electrical adapter with a lock mechanism — explicitly including a 'combination-type' lock — that both controls power flow AND physically locks a plug to the adapter's female receptacle. It directly addresses the concept of a code-based release mechanism on a plug security device (KF 3), and combines physical plug locking with power-disable functionality in an adapter form factor.

| Patent No. | US5911586A (https://patents.google.com/patent/US5911586A) |
| --- | --- |
| Title | System, Device and Method for Locking and Unlocking Power Flow to an Electrical Cord |
| Filing Date | May 30, 1997 |
| Publication Date | June 15, 1999 |
| Earliest Priority | June 1, 1996 (Provisional US1957496P) |
| Inventor(s) | H. Peter Wintergerst |
| Assignee — Original | H. Peter Wintergerst (Individual) |
| Family Members | US1957496P (provisional) |
| CPC / IPC | H01R13/70 ¦ H01R13/6397 ¦ H01R31/06 ¦ H01R31/065 ¦ H01R13/6392 |

Relevant Excerpts:

• An adapter "by using a lock and key mechanism, will enable or disable power from flowing through said power cord or adapter."  ~Abstract

• The locking mechanism can be "a simple lock and key, a combination-type lock, or even an electronic lock requiring a password code."  ~Detailed Description

• "One is prevented from initiating power flow without access to the first key means, and from removing the plug without access to the second key means."  ~Claims

• The device describes "locking and unlocking a plug to and from the female receptacle" to prevent unauthorized use.  ~Abstract

• The adapter form factor has "a male plug end inserted into female power source and a female receptacle end which receives the plug."  ~Claims

• Purpose is "to prevent unauthorized use of any electrical device" by both "disabling a flow of current" and "locking the plug."  ~Abstract

## Reference 3 — US7497705B2

This Master Lock patent discloses an industrial plug lockout device with an axially movable retaining member that engages through transverse apertures in plug prongs. The retaining member automatically engages upon prong insertion and can be secured with a padlock. While designed for industrial LOTO, the automatic pin engagement through prong apertures parallels ReBolt's telescopic pin concept (KF 2, KF 2A), and the lockout-preventing-reinsertion function maps to KF 2B.

| Patent No. | US7497705B2 (https://patents.google.com/patent/US7497705B2) |
| --- | --- |
| Title | Lockout Device |
| Filing Date | September 12, 2007 |
| Publication Date | March 3, 2009 |
| Earliest Priority | September 15, 2006 (Provisional US84482506P) |
| Inventor(s) | Dwayne Larson, Michael Brojanac, Joe Elliott, Mark D. Johnson |
| Assignee — Original | Master Lock Co LLC |
| Family Members | CA2663020A1 ¦ WO2008034055A2 ¦ MX2009002823A ¦ CN101536265A ¦ TW200818619A |
| CPC / IPC | H01R24/20 ¦ H01R13/6397 ¦ H01R2103/00 |

Relevant Excerpts:

• A lockout device with "a retaining member disposed within the body and axially movable between a prong retaining position and a prong releasing position."  ~Abstract

• The retaining member includes "a prong engaging portion configured to engage at least one of the first and second prongs when the first prong is inserted."  ~Abstract

• "A prong retaining portion configured to extend through the first transverse aperture" in the prong when in the retaining position.  ~Claims

• The retaining member is a "single-piece spring wire with hooks at each end that extend laterally across the slots and through the prong apertures."  ~Detailed Description

• "Insertion of plug prongs automatically moves the retaining member from releasing to retaining position — no additional manipulation needed."  ~Detailed Description

• A lock opening "sized to receive a lock member for preventing movement of the retaining member from retaining to releasing position."  ~Claims

## Reference 4 — US10483694B2

This patent discloses a lock-in adapter with internal locking arms ending in latches that secure a plug inserted into the adapter AND secure the adapter itself into a wall outlet via a ground-pin latch. A single button releases both locks. The adapter-between-plug-and-outlet concept, the internal latching mechanism, and the single-action release are all relevant to ReBolt's adapter architecture (KF 1, KF 4) and release mechanism (KF 3).

| Patent No. | US10483694B2 (https://patents.google.com/patent/US10483694B2) |
| --- | --- |
| Title | Lock-in Securable Electrical Plug Adapter and Method of Use |
| Filing Date | December 28, 2018 |
| Publication Date | November 19, 2019 |
| Earliest Priority | October 4, 2017 (CIP of US10181680B1) |
| Inventor(s) | Robert W. Crowder |
| Assignee — Original | Robert W. Crowder (Individual) |
| Family Members | US10181680B1 (parent) ¦ US15/725,224 |
| CPC / IPC | H01R13/6395 ¦ H01R13/447 ¦ H01R13/582 ¦ H01R13/5825 ¦ H01R31/06 ¦ H01R31/065 ¦ H01R24/66-78 |

Relevant Excerpts:

• An adapter with "a locking arm ending in a latch contained within a ground pin for securing the adapter into a wall outlet."  ~Abstract

• "Internal locking arms similarly lock and retain a plug inserted into the opposite end of the adapter, thereby securing the plug to the adapter and the adapter to the wall."  ~Abstract

• "A button secures the various latches... and which can be selectively operated to release both the adapter from the outlet and the plug from the adapter."  ~Abstract

• Latches have "tapered front ends allowing objects to pass by, then springs force latches back into locking position."  ~Detailed Description

• The ground prong contains a "cavity containing an arm ending in a latch that grabs onto the rear face of the outlet, preventing removal."  ~Detailed Description

• "A removable button clip can be placed beneath the button to prevent accidental depression" of the release.  ~Detailed Description

## Reference 5 — US7077683B1

This patent discloses a ratcheting mechanism on electrical plug prongs — serrated/sawtooth edges on prong pins that cooperate with a spring-biased retaining member in a receptacle. The ratcheting engagement is automatic upon insertion and requires deliberate manual action to release. This is directly relevant to ReBolt's ratcheting clamp mechanism (KF 1A) and the concept of a one-way mechanical engagement on plug prongs.

| Patent No. | US7077683B1 (https://patents.google.com/patent/US7077683B1) |
| --- | --- |
| Title | Ratcheting Electrical Plug and Receptacle |
| Filing Date | June 15, 2005 |
| Publication Date | July 18, 2006 |
| Earliest Priority | June 15, 2005 |
| Inventor(s) | Nicky Charles Ross |
| Assignee — Original | Nicky Charles Ross (Individual) |
| Family Members | US application only |
| CPC / IPC | H01R13/443 ¦ H01R13/20 ¦ H01R13/6392 ¦ H01R13/6395 ¦ H01R13/6272 ¦ H01R13/635 |

Relevant Excerpts:

• An apparatus with "a serrated edge on at least one pin of the plug that cooperates with a retaining member in the receptacle."  ~Abstract

• "Linear insertion of the plug allows a portion of the retaining member to engage with at least one sawtooth of the serrated edge sufficient to secure the plug thereto."  ~Abstract

• The sawtooth profile creates one-way engagement: "the leading incline edge contacts the retaining member incline, pushing it upward" while "the flat edge bears against the flat portion, preventing withdrawal."  ~Detailed Description

• "Continued insertion repeats for each sawtooth, progressively securing the plug" — a ratcheting, incremental locking mechanism.  ~Detailed Description

• Release requires user to "press down on rear portion of retaining member to compress the spring and raise it above the serrated edge."  ~Detailed Description

• "No rotation required for either insertion or removal" — securement is automatic upon linear insertion.  ~Detailed Description

## Reference 6 — US5071360A

This patent is among the most relevant to ReBolt. It discloses a permanently attachable adapter with a locking pin inserted through prong holes to create a permanent, non-reversible bond between the adapter and the appliance plug. The pin uses barbed plastic that collapses on insertion and expands to prevent removal. Combined with a key switch for power control, this directly anticipates ReBolt's permanent attachment (KF 1, KF 1C) and the concept of a pin mechanism associated with prongs (KF 2).

| Patent No. | US5071360A (https://patents.google.com/patent/US5071360A) |
| --- | --- |
| Title | Permanently Attachable Key-Activated On/Off Switch |
| Filing Date | May 17, 1990 |
| Publication Date | December 10, 1991 |
| Earliest Priority | October 4, 1988 (Continuation of US4969833A) |
| Inventor(s) | Edgar J. Lindow, Denise L. Domning |
| Assignee — Original | Edgar J. Lindow / Denise L. Domning (Individuals) |
| Family Members | US4969833A (parent) |
| CPC / IPC | H01R13/6395 ¦ H01R13/66 ¦ H01R13/70 ¦ H01R13/6397 |

Relevant Excerpts:

• A "permanently attachable" device with "a locking pin that is inserted through the prongs of the receptacle and an appliance plug in the receptacle so that the appliance plug is permanently attached."  ~Abstract

• The locking pin is a "plastic pin with barbed tip" that "collapses to pass through the holes then expands on the far side, permanently preventing pin removal."  ~Detailed Description

• "The pin head sits flush in a countersunk hole, preventing any pulling force" — permanent, non-reversible attachment.  ~Detailed Description

• "Standard UL-approved plugs already have holes in their prongs" which the pin exploits for alignment and engagement.  ~Detailed Description

• A "key-operated SPST switch controls power flow independently of the locking pin" — combining physical lock with power control.  ~Detailed Description

• "The appliance plug is then permanently attached to the device" and "the device attaches to the appliance, not the wall, allowing mobility."  ~Detailed Description

## Reference 7 — US11824309B2

This recent patent discloses a locking electrical receptacle with clamping jaws that lock a plug in place, AND that blocks the socket apertures when no plug is inserted — preventing reinsertion by unauthorized users. The automatic blocking-when-unplugged function maps to ReBolt's auto-extending pin concept (KF 2A, KF 2B). It also discloses a security button required before the lock can be operated (KF 3).

| Patent No. | US11824309B2 (https://patents.google.com/patent/US11824309B2) |
| --- | --- |
| Title | Device and Method for Locking Electrical Plugs to Power Outlets |
| Filing Date | May 7, 2018 |
| Publication Date | November 21, 2023 |
| Earliest Priority | May 9, 2017 |
| Inventor(s) | Mohamed Abouismail |
| Assignee — Original | Mohamed Abouismail (Individual) |
| Family Members | CN110710063B ¦ WO2018/207079 ¦ EA041720B1 ¦ EP ¦ JP ¦ AU ¦ CA |
| CPC / IPC | H01R13/6395 ¦ H01R13/04 ¦ H01R13/05 ¦ H01R13/193 ¦ H01R13/447 ¦ H01R13/6392 |

Relevant Excerpts:

• A device that "connecting and locking electrical plugs into power outlets" and that "blocks the socket openings to prevent children from tampering."  ~Abstract

• "When no plug is inserted and locked, the clamping mechanism blocks the apertures" — automatic blocking upon plug removal.  ~Claims

• Clamping jaws with "friction pads (rubber/silicon)" apply symmetric force to "prevent bending of prongs."  ~Claims

• An oval cam controls jaw position: "when rotated 90 degrees... jaws compress and clamp plug prongs."  ~Detailed Description

• "Security button must be pressed before rotating sleeve to unlock" — two-step authorization for release.  ~Detailed Description

• Multiple embodiments for "household, medical devices, computers, factory equipment, and home appliances."  ~Abstract

## Reference 8 — US6419504B1

This patent discloses a power plug with a spring-biased, retractable grounding pin that extends automatically from the plug body when released and retracts when a slide lock is actuated. While designed for ground-pin retraction (for 2-prong outlet compatibility), the mechanism of a spring-loaded, automatically extending pin with a slide-lock control is directly relevant to ReBolt's telescopic pin concept (KF 2, KF 2A, KF 2C).

| Patent No. | US6419504B1 (https://patents.google.com/patent/US6419504B1) |
| --- | --- |
| Title | Slide Locked Retractable Grounding Pin Power Cord Plug |
| Filing Date | April 17, 2001 |
| Publication Date | July 16, 2002 |
| Earliest Priority | April 17, 2001 |
| Inventor(s) | Richard Bryant Nelson |
| Assignee — Original | Richard Bryant Nelson (Individual) |
| Family Members | WO (PCT/US2002/012220) |
| CPC / IPC | H01R24/70 ¦ H01R13/648 ¦ H01R13/652 ¦ H01R29/00 ¦ H01R2103/00 |

Relevant Excerpts:

• A plug with "a grounding pin which is slidingly mounted within a recess... positionable between an extended position... and a retracted position fully enclosed within the recess."  ~Abstract

• "A biasing spring is positioned within the recess for urging the grounding pin into the extended position" — automatic extension.  ~Abstract

• "A locking device includes a finger actuatable slide" that moves a "tongue into a locking position inhibiting retraction of the grounding pin."  ~Abstract

• The slide lock engages a "tongue finger" into a "slot" in the grounding pin, locking it in extended position.  ~Claims

• "When slide is pulled back, tongue resumes non-deflected position, pin is free to retract" — user-controlled release.  ~Detailed Description

• The device "can also be formed as a separate adapter with a plug socket" — adapter form factor.  ~Detailed Description

## Reference 9 — Master Lock S2005 Compact Plug Prong Lockout (NPL)

The Master Lock S2005 is a commercially available industrial lockout device that directly grips the prongs of 110-120V plugs, blocking insertion into any outlet. It accepts a padlock shackle for security. As a commercial product, it constitutes prior art for prong-blocking plug lockout concepts (KF 2B). However, it is removable (not permanent), uses a separate padlock (not a built-in combination), and has no automatic deployment mechanism.

| Title | Master Lock S2005 — Compact Plug Prong Lockout |
| --- | --- |
| Publication Date | Current commercial product (Active) |
| Author / Manufacturer | Master Lock Co LLC |
| Publisher / Source | masterlock.com/products/product/S2005 |

Relevant Details:

• A "patented plug lockout device" that "grips prongs of 110-120 volt plugs" — direct prong engagement for lockout.  ~Product Description

• Fits "two prong plugs with and without ground" — universal plug compatibility.  ~Product Specifications

• Accepts "3/16in, 1/4in, and 9/32in shackle diameters for use with existing padlocks/hasps" — separate lock required.  ~Product Specifications

• Designed for industrial LOTO (Lockout/Tagout) safety compliance per OSHA/NFPA 70E standards.  ~Product Context

## Reference 10 — Lock Socket Consumer Charger Security Device (NPL)

Lock Socket is a commercially successful consumer product (300,000+ units sold, 5,500+ facilities) that secures phone charger blocks to wall outlets to prevent theft. Associated patent US11557864B2 (Varjabedian). While addressing the identical market problem as ReBolt (charger anti-theft), Lock Socket secures the charger to the outlet rather than attaching permanently to the plug prongs. It has no telescopic pin, no combination code, and no prong-blocking mechanism.

| Title | Lock Socket — Charger Security Device |
| --- | --- |
| Publication Date | Current commercial product (Active, 2019-present) |
| Author / Manufacturer | Lock Socket Inc. / Shaant Varjabedian |
| Publisher / Source | locksocket.com |

Relevant Details:

• "Secures phone charger blocks to wall outlets" — consumer anti-theft for chargers in public spaces.  ~Product Description

• Over "300,000+ units sold" in "5,500+ facilities" including "hospitals, arenas, venues."  ~Product Marketing

• Constructed from "Fire Resistant ABS (CYCOLAC FR15U-NA1000)" — safety-rated material.  ~Product Specifications

• Multiple SKUs for different charger types — iPhone, iPad, Samsung, USB-C configurations.  ~Product Catalog

# Section 7 — Other Peripheral Prior Art References

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

## 7.1 Patent Citations

| S.No. | Publication No. | Title | Assignee(s) |
| --- | --- | --- | --- |
| 1 | US5551884A | Locking Electrical Outlet | George D. Burkhart (Individual) |
| 2 | US6171129B1 | Locking Electrical Adapter | Gerald L. Phillips (Individual) |
| 3 | US6875040B1 | Lockable Electric Power Cord Adapter | Piranha Plugs Inc. |
| 4 | US6979220B1 | Plug Locking Mechanism | Card Access Inc. |
| 5 | US6773277B2 | Security Cap for Electrical Plugs | Safi Issa (Individual) |
| 6 | US10326241B2 | iCubeLock — Charger Lock Bracket | Ira Sheldon Jacobson (Individual) |
| 7 | US10998676B2 | Frictional Locking Receptacle with Programmable Release | Zonit Structured Solutions LLC |
| 8 | US8215972B2 | Anti-tamper Adapter Blocking Plug Release | Leviton Manufacturing Co. |
| 9 | EP4560842A1 | Access Control Assembly for Plug End | The Boeing Company |
| 10 | US3539968A | Electrical Plug Blade Safety Cover | Wilfred Tunstall (Individual) |

## 7.2 Non-Patent Citations

| S.No. | Title | Author(s) | Publication Date |
| --- | --- | --- | --- |
| 1 | Master Lock 487 — Rotating Electrical Plug Lockout | Master Lock Co LLC | Current |
| 2 | Brady PLO23 — 3-in-1 Plug Lockout | Brady Corporation | Current |
| 3 | iCubeLock — Commercial Charger Lock Product | iCubeLock | 2018+ |
| 4 | NFPA 70E §120.2-120.5 — Lockout/Tagout Program Requirements | NFPA | 2021 |

# Section 8 — Claim Differentiation and Design-Around Strategy

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

## 8.1 Overlap Matrix

| Key Feature | Ref 1 US4579410 | Ref 2 US5911586 | Ref 3 US7497705 | Ref 4 US10483694 | Ref 5 US7077683 | Ref 6 US5071360 | Ref 7 US11824309 | Ref 8 US6419504 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KF 1: Permanent adapter-to-plug attachment | Partial | No | No | No | No | Full | No | No |
| KF 1A: Ratcheting clamp (zip-tie style) | No | No | No | No | Partial | No | No | No |
| KF 1B: Medical/industrial adhesive | No | No | No | No | No | No | No | No |
| KF 1C: Irreversible without destruction | Partial | No | No | No | No | Full | No | No |
| KF 2: Telescopic pin in adapter | No | No | Partial | No | No | Partial | No | Partial |
| KF 2A: Auto-extends on unplug | No | No | Partial | No | No | No | Partial | Partial |
| KF 2B: Blocks prong faces | Partial | No | Partial | No | No | No | Partial | No |
| KF 2C: Retracts only with user action | No | No | Partial | Partial | Partial | No | Partial | Partial |
| KF 3: Combination code release | No | Partial | No | No | No | No | No | No |
| KF 4: Features 1+2 combined | No | No | No | No | No | No | No | No |
| KF 5: Universal plug compatibility | Partial | Partial | Partial | Partial | No | Partial | Partial | No |

## 8.2 Whitespace Analysis

KF 1B (Medical-grade/industrial adhesive as a bonding mechanism between adapter and charger body): No identified prior art reference discloses the use of adhesive bonding as part of a permanent plug-adapter attachment system. While adhesives are widely used in manufacturing and assembly, their specific application as a complementary bonding agent alongside a mechanical clamp to permanently affix a security adapter to a charger body appears to be entirely novel in the electrical plug security field. This represents a clear area of whitespace.

KF 1A (Ratcheting/zip-tie style clamp on prong base): While US7077683B1 (Ross) discloses a ratcheting serrated mechanism on plug prongs, that patent applies the ratcheting to the prong-to-receptacle interface (not adapter-to-plug), and uses it for retention rather than permanent attachment. No reference discloses a zip-tie-style irreversible ratcheting clamp that cinches around the base of existing plug prongs to permanently bond a third-party adapter. This sub-feature appears to be well-differentiated.

KF 2A combined with KF 2B (Telescopic pin that auto-extends upon unplugging to block prong faces): While several references disclose retractable pins (US6419504B1) and prong-blocking devices (US3539968A, US7497705B2), no identified reference combines automatic deployment triggered by the act of unplugging with prong-face blocking. US6419504B1's pin is spring-biased to extend but for ground-pin functionality, not security blocking. US11824309B2 blocks socket apertures when unplugged, but from the outlet side, not from a plug-mounted adapter. The specific mechanism of a pin emerging from an adapter face upon unplug to cover prong contact surfaces appears novel.

KF 3 combined with KF 2 (Combination code controlling telescopic pin retraction): US5911586A (Wintergerst) discloses a combination-type lock on a plug adapter but for controlling power flow, not for retracting a physical pin. No reference combines a combination code input with the specific function of retracting a telescopic prong-blocking pin. This combination of mechanical pin actuation governed by a combination code input appears to be the strongest point of novelty in the ReBolt design.

## 8.3 Suggested Claim Differentiation Language

The following example claim limitation phrases are offered as starting points for patent claim drafting. These focus on the areas of whitespace identified above.

1. "...wherein the adapter is permanently affixed to the male prongs of the plug by a combination of (a) a ratcheting mechanical clamp that cinches around the prong base in a single direction and (b) an adhesive applied to a mating surface between the adapter body and the plug body, such that the adapter cannot be removed without destruction of at least one of the adapter or plug..."

2. "...wherein a telescopic pin housed within the adapter body is biased to an extended position and automatically extends from the adapter face upon withdrawal of the plug from an electrical outlet, the extended pin physically obstructing the contact surfaces of the plug prongs to prevent re-insertion into any electrical outlet..."

3. "...wherein the telescopic pin retracts from the extended prong-blocking position only upon successful entry of a predetermined combination code via a user-operable combination mechanism integrated into the adapter body..."

4. "...wherein the ratcheting clamp comprises a plurality of angled teeth that permit incremental tightening around the prong base in a first direction while preventing loosening in a second direction, in the manner of a cable tie or zip-tie mechanism..."

5. "...wherein the adapter, the permanent attachment mechanism, the telescopic pin, and the combination release mechanism are integrated into a single unitary device mountable on any standard AC power plug or charger block without modification to the plug..."

Disclaimer: This is not legal advice and is not a substitute for claim drafting by a registered patent practitioner. These are starting-point suggestions for further refinement.

# Section 9 — Search Strategy Disclosure

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

## Pass 1 — Keyword Queries (10 queries executed)

1. plug lock adapter permanent attachment prong security

2. ratchet clamp power plug security device adhesive

3. charger lock anti-theft adapter plug prong

4. power cord lock plug retention device permanent

5. zip tie ratchet mechanism electrical plug clamp

6. telescopic pin plug adapter block insertion security

7. spring loaded pin electrical plug lock combination

8. power plug lockout device retractable pin code

9. outlet plug lock combination dial mechanism

10. electrical plug disable block prong face cover lock

## Pass 2 — CPC/IPC Classification Codes Searched

• H01R13/6397 — Means for preventing unauthorised use

• H01R13/6395 — Holding/locking for wall or panel outlets

• H01R31/06 — Intermediate connectors (adapters)

• H01R13/70 — Structural association with built-in switch

• H01R13/20 — Pins/blades shaped to retain co-operating parts

• H01R13/6392 — Holding/locking for extension cord

• H01R13/627 — Latching means integral with housing

• H01R13/443 — Dummy plugs

• H01R24/20 — Coupling parts carrying sockets

• E05B73/00 — Locking portable objects against unauthorised removal

## Pass 3 — Citation Chaining Anchor References

• US4579410A (Soloman) — 5 backward, 30 forward citations examined

• US10483694B2 (Crowder) — 35 backward, 19 forward citations examined

• US5911586A (Wintergerst) — 6 backward, 12 forward citations examined

• US7497705B2 (Master Lock) — 22 backward, 19 forward citations examined

• US11824309B2 (Abouismail) — 89 backward, 9 forward citations examined

Foundational patents appearing in 2+ citation lists: US5551884A (3 lists), US5071360A, US6171129B1, US7077683B1, US7798838B2, US5791931A, US6193539B1, US20030207606A1, US20140213093A1, US4494809A.

## Pass 4 — Foreign Jurisdictions & NPL Sources

Foreign jurisdictions searched: CN (China), JP (Japan), KR (Korea), DE (Germany), EP (Europe), GB (Great Britain), SE (Sweden), FR (France), EA (Eurasian)

Key foreign references identified: CN110710063B, JP5486398B2, KR200243901Y1, KR200284651Y1, DE102021117785A1, GB2158662A, SE544579C2

NPL sources searched: Google Scholar (blocked by CAPTCHA), IEEE Xplore (blocked by CAPTCHA), arXiv (no relevant results), Master Lock product catalog, Brady Safety product catalog, Lock Socket commercial product, iCubeLock commercial product, NFPA 70E standards, Amazon, Grainger, Kickstarter, Indiegogo

# Section 10 — Disclaimer

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

This patentability search report was conducted with reasonable care and diligence using publicly available patent and non-patent databases at the time of the search (April 2026). While multiple search passes were employed — including keyword searches, CPC/IPC classification mining, citation chaining, and foreign/NPL literature review — completeness of the search cannot be guaranteed.

Patent databases are subject to publication delays (typically 18 months from filing), and recently filed applications may not yet be publicly available. Paid commercial databases (Orbit Intelligence, PatSnap, Derwent Innovation, TotalPatent One) and native-language searching by local patent professionals may surface additional foreign-language references not identified in this search.

This report does not constitute a legal opinion on the patentability, novelty, or non-obviousness of the disclosed invention. No legal conclusions are rendered herein. The inventor is strongly advised to consult a registered patent attorney or patent agent before making any filing decisions based on this report.

All patent excerpts are paraphrased or limited to brief quotations (under 15 words) for fair use purposes. Full patent documents should be reviewed in their entirety by the patent practitioner.

The overlap assessments, whitespace analysis, and claim differentiation suggestions in Section 8 are provided as informational starting points only and are not a substitute for professional claim drafting by a registered patent practitioner.